## Accessible Meeting Facilities Checklist

## Governor’s Committee on Disability Issues and Employment (GCDE)

## Instructions:

This checklist is to be used by all state agencies to determine if a meeting space is accessible and is based on new construction standards.  While older facilities may lack some of these items, asterisked items are essential in making your selection of the meeting place.

This checklist has been updated to include ADA Accessibility Guidelines (ADAAG), WAC 51-50 (Washington State Barrier-Free Code, including the new International Building Code) and ANSI 117.1.

A separate checklist is required for each facility and should be kept with the meeting materials*. This way if the agency uses a particular site frequently, the completed checklist can be reused. Some pages of the checklist will be used more than once; others will not be needed at all. If the element does not exist, mark not applicable or “N/A”. The vendor is solely responsible to ensure that the non-state facility meets the minimum accessibility requirements contained in these regulations, and that any special modifications to the non-state facility are completed for the event.

OFM State Administrative and Accounting Manual Provisions:

50.50.70.5	Ensure the non-state facility vendor attaches a signed ADA certification statement and a completed Accessible Meeting Site Survey Form to the invoice being submitted to the state agency for payment.

50.50.70.6	The ADA certification statement is to read: “To the best of (vendor's name) knowledge, the facilities provided to the (state agency's name) on (date or dates) met all of the minimum accessibility requirements (and any special modifications to the facilities included in the contract).  It is not necessary for the vendor to attach a completed copy of the Accessible Meeting Facility Checklist to the invoice, if the agency has a current copy of it on file and references the checklist and the date of completion on the face of the invoice.”

50.50.80.a	Should a state agency obtain a receiving report with a statement by its representative indicating that the non-state facility did not meet ADA access criteria as identified in this document, the agency is authorized to deny payment to the vendor due to breach of contract.

The principle of universal access is central to any accessibility review.  All services should be provided in an integrated setting, separate is not equal.  In this checklist an accessible route of travel means a path 36" wide (interior), 44" (exterior) without abrupt changes greater than 1/4". The route should be one path accessible by everyone.  The route provides a smooth, hard surface with access from parking to primary building entrance(s), through the interior of the building to meeting rooms, restrooms, restaurants, including coffee bars, lobbies and shops.

| Name of Facility (hotel, restaurant, city or county facility): |  |
| --- | --- |
| Address: Phone #: | City: Zip Code: |
| Surveyor’s Name: Phone #: | Survey Date: Office/Agency: |

* You may send an electronic copy of the checklist to Carol Maher, Barrier-Free Facilities Program, at General Administration, cmaher@ga.wa.gov  so they can be shared with other agencies. Paper copies may be sent to: GCDE - This may assist planners in locating accessible meeting sites in the future. If you find an exceptional place to meet, please share that information also. Checklists can be mailed to the Governor’s Committee on Disability Issues and Employment, PO Box 9046, MS: 6000, Olympia, WA 98507-9046.

# Building Exterior

## Off-Street Parking/Passenger Loading Zone

|  | Number of accessible parking spaces, (see Appendix A) # Required Spaces |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | *At least one van space, 96"space with 96"access aisle/132"space with 60", 114"vertical clearance) |  |  |  |  |  |  |  |
|  | Sign at parking space, International Access Symbol, white on blue, “Van Accessible” for van space |  |  |  |  |  |  |  |
|  | Built-up curb ramps do not project into access and parking spaces. |  |  |  |  |  |  |  |
|  | *Level Slope < 1:48, firm, and non-slip surface. Slope < 1:48 |  |  |  |  |  |  |  |
|  | *Curb-cut, ramp or level area to walkway |  |  |  |  |  |  |  |
|  | *If surface unpaved, then size of gravel < 0.3 inches |  |  |  |  |  |  |  |
|  | Closest parking space to accessible entrance. Crosses vehicular traffic lane? | Y |  |  |  |  | N |  |
|  | Directional signage to accessible entrance, at non-accessible entrance |  |  |  |  |  |  |  |
|  | Passenger drop off or loading zone with accessible route or travel to building |  |  |  |  |  |  |  |

## Proximity to Public Transportation

|  | Bus stop within 1-2 blocks. Approximate Distance |  | Bus available evenings | Y |  |  | N |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | * Level, firm, non-slip surface from bus stop to primary accessible building entrance, maximum slope of 1:12 |  |  |  |  |  |  |  |
|  | *Curb cut, ramp, or level area to walkway (see Walkways). Note: handrails required if slope > 1:20 and the rise is >6" |  |  |  |  |  |  |  |

Walkways

|  | 44" minimum exterior width |
| --- | --- |
|  | *Max. slope of 1:12 (Up to 1:8 permissible for rises < 3" Up to 1:10 for rises < 6") preferably 1" to 20" (can carry a latte in your lap and go down ramp without spilling) |
|  | Level (slope >1:20) or ramped from parking to primary accessible entrance |
|  | Level, firm, non-slip surface with no drop-offs, grass or soil meet sidewalk |
|  | Walkways free of obstructions that protrude > 4" (higher than 27"or < 80") |
|  | Walkways free of grating openings larger than 1/2", openings perpendicular to path of travel. |
|  | Threshold 1/4" maximum, or 1/2" if beveled |

*Essential for an accessible meeting

Building Exterior (Cont’d)

## Ramps (exterior)

|  | Maximum slope of 1':12' (no more than 30' between landings); slope 1':20' (40' between landings) |
| --- | --- |
|  | *Landings at top and bottom of run, Landings shall be level and be 60"in direction of travel |
|  | *Graspable handrails provided, 34"- 38" high (Slope 1:20, or rise < 6" no handrails required.) |
|  | Handrails 1-1/2" diameter and 1-1/2" from wall |
|  | Firm, non-slip surface |
|  | 44" minimum exterior width |

## Stairways

|  | *Graspable handrails provided on both sides, 34" - 38" high, properly secured |
| --- | --- |
|  | Handrails 1-1/2" diameter and 1-1/2" from wall |
|  | Uniform riser height and tread width |
|  | 5' x 5' level landings on top and bottom |
|  | Contrast on stairs and landings |
|  | Adequate lighting on stairs |
|  | *No open risers (steps). No hanging stairwells, unless cane detectable barriers are provided underneath. |

## Entrances (exterior)

|  | *At least one primary entrance accessible, door 32"clear opening |  |  |
| --- | --- | --- | --- |
|  | Threshold height 1/4" maximum, 1/2" if beveled. If not, actual height is |  |  |
|  | An 18" clear maneuvering space at the pull side of the door |  |  |
|  | Level and unobstructed area 5' x 5' both sides of door |  |  |
|  | Lever or loop-type door handles |  |  |
|  | Door opening pressure 8.5 lbs. maximum, or Automatic door openers |  |  |
|  | *Alternate accessible entrance for a revolving door |  |  |
|  | Sign indicating accessible entrance |  |  |
|  | Directional signage at inaccessible entrances designating the accessible entrance |  |  |

*Essential for an accessible meeting

# Building Interior

## Interior Doors and Corridors

|  | *Firm, non-slip surface (no loose or deep pile carpet, maximum pile thickness < 1/2") |
| --- | --- |
|  | *Doors have a minimum clear opening width of 32" |
|  | *An 18" clear maneuvering space at the pull side of the door |
|  | *Lever or loop-type handles, path to meeting room |
|  | *Door pressure 5 lbs. Maximum, or Automatic door |
|  | *Corridors have a clear width of 36" |
|  | Wall-mounted objects protruding 4" or greater (located within 27" - 80" from the floor) have barriers detectable by individuals using a white cane. Wall mounted objects protruding less than 4" or higher than 80"from the floor, no detectable barrier required |
|  | Adequate lighting in corridors, provide uniform illumination |

## Ramps (interior)

|  | *Maximum slope of 1':12' (no more than 30' of rise between level landings) |
| --- | --- |
|  | *5' x 5' level landings on top and bottom |
|  | *Graspable handrails provided, 34" - 38" high |
|  | *Handrails 1-1/2" diameter and 1-1/2" from wall |
|  | *Firm, non-slip surface |
|  | *36" minimum interior width |

## Elevators

|  | *Door has 36" minimum clear opening |
| --- | --- |
|  | *Size of elevator floor at least 54" x 68" |
|  | *Serves all floors and public meeting areas |
|  | *Highest control buttons 48" maximum (54" built before 2002), emergency controls 35" |
|  | *Audible and visible signals, hallway and elevator interior |
|  | Controls have raised Arabic numerals and Braille identification |
|  | Exterior call buttons 35"max |
|  | Floor levels indicated on door jambs by raised numerals placed no more than 60" high |
|  | Elevator doors remain fully open for 5 seconds minimum |
|  | *Reopening device that will reopen a car door automatically if the door becomes obstructed |
|  | *Visible and audible signal provided at each entrance to indicate which car is answering a call |

*Essential for an accessible meeting

# Building Interior (Cont’d)

## Public Telephones (where provided)

|  | *At least one telephone/floor usable wheelchair accessible open space in front at least 30" by 48" |
| --- | --- |
|  | *Highest operable part 48" maximum side or forward approach |
|  | 27" high clear knee space |
|  | Handset cord length at least 29" |
|  | All public telephones are required to have volume control |
|  | TTY in bank of 4 or more |

## Water Fountains (where provided)

|  | *At least one fountain on accessible route of travel |
| --- | --- |
|  | Maximum spout no higher than 36" from floor |
|  | Spout located at front of unit with water projecting parallel |
|  | Hand operated control (push or lever) within 5" of the front of the fountain |
|  | 27" clear knee space |
|  | If no knee space, then at least 30" x 48" clear floor space provided for parallel approach *Essential for an accessible meeting |

*Essential for an accessible meeting

# Public Restrooms

## Women	Men 	(One restroom may be accessible while another is not, check both)

|  |  |  |  | *On accessible route of travel from or to meeting room |
| --- | --- | --- | --- | --- |
|  |  |  |  | *At least one accessible stall in each restroom. Or unisex restroom available |
|  |  |  |  | Ambulatory accessible toilet stalls (required when six or more water closets are available in a restroom) |
|  |  |  |  | *High contrast, non-glare sign, raised and Braille between 48" - 60" from floor, located on latch side of door |
|  |  |  |  | Signs at inaccessible restrooms giving directions to accessible restrooms |
|  |  |  |  | *Entry 32" minimum clear width |
|  |  |  |  | *Accessible stall doors 32" minimum clear width |
|  |  |  |  | *Door pressure 5 lbs. maximum |
|  |  |  |  | *Stall width 60" wide x 56" wall mounted, 60" x 59"floor mounted toilet |
|  |  |  |  | Minimum 48" width next to toilet on one side |
|  |  |  |  | *Grab bars side and back, 33" - 36" above and parallel to floor |
|  |  |  |  | Grab bars 1-1/2" diameter and 1-1/2" from wall |
|  |  |  |  | *Toilet seat 17" - 19" high |
|  |  |  |  | 5' x 5' diameter clear floor space to turn around (by mirrors or sink area) |
|  |  |  |  | Bottom of mirror, top of shelf, towel and all other types of dispensers at 40" maximum from floor |
|  |  |  |  | Soap and towel dispensers and hand dryer adjacent to the sink |
|  |  |  |  | 27" clear knee space under basin |
|  |  |  |  | Insulation of exposed pipes under sinks |
|  |  |  |  | *Lever-type faucets (or automatic) |
|  |  |  |  | * Lever-type door hardware, entry door and on accessible stall and urinal |
|  |  |  |  | Elongated urinal within 17" of floor |

*Essential for an accessible meeting

# Meeting Rooms and Common Use Areas

## Meeting Rooms – Room # / Name of room: __________________________________

(please complete for each meeting room to be used)

|  | *Capacity |
| --- | --- |
|  | High contrast signage with non-glare finish, Raised and Braille at 48"- 60", latch side of door |
|  | *Ramps for raised platforms, speaking areas |
|  | Top of table 28" - 34" from floor |
|  | Clear knee space for tables (minimum 27" high x 30" wide x 19" deep) |
|  | Public Address System with Assistive Listening equipment |
|  | Assistive Listening Equipment (identified by signage) |
|  | Low noise level (inside and outside) |
|  | Uniform lighting |
|  | *Meeting and other functions provided in nonsmoking areas |
|  | *Firm, non-slip surface (no loose or deep pile carpet) |
|  | *If Audible, then visible alarm system |

## Fixed Seating Only (auditorium)

|  | For auditoriums, integrated wheelchair seating, a minimum of one, for 4-25 seats |
| --- | --- |
|  | Number of wheelchair spaces required _________ (See Appendix A Wheelchair Spaces Required in Assembly Areas) |
|  | Minimum space 33" x 48" for rear or forward access, 33" x 60" for side access |
|  | *Unobstructed viewing position from wheelchair seating |
|  | Aisles at least 36" having seating on one side of aisle, 42" with seating on both sides |
|  | Integrated seating, people using wheelchairs can sit next others, accessible seating dispersed through out auditorium. |

*Essential for an accessible meeting

# Meeting Rooms and Common Use Areas (Cont’d)

## Common Use Areas

|  | *Restaurant /coffee shops, gift shops, ATM, lobby, vending machines, copy machines and other common use areas accessible to persons with disabilities. (entrance, seating, counter height, reach range, 48" ) |
| --- | --- |

| Problem Areas: |  |
| --- | --- |

|  | Audible alarm system |
| --- | --- |
|  | Visible alarm system |
|  | Smoking policy or restrictions in common use areas |
|  | Maintenance/remodeling at time of meeting |
|  | Swimming Pool Access |

*Essential for an accessible meeting

# Hotel/Motel Guest Rooms

## Hotel/Motel Guest Rooms

|  | Number of standard guest rooms |
| --- | --- |
|  | *Number of accessible guest rooms (Accessible hotel or motel guest rooms should be provided at the rate of one for every 25 guest rooms or fraction thereof. Rooms should be dispersed in single, double and suite rooms.) |
|  | *Number of accessible rooms with roll-in showers |
|  | Number of accessible guest rooms located near the elevators |
|  | Number of nonsmoking guest rooms |
|  | *Raised and Braille signage 48" to 60" above floor on latch side of door |
|  | *High contrast signage with non-glare finish |

## Wheelchair Accessible Rooms

|  | *Entry 32" minimum clear opening width |
| --- | --- |
|  | *Lever-type hardware on doors |
|  | * Telephone and other controls easily reached from bed and accessible route of travel |
|  | Closet rods, switches, heat and air conditioning controls within reach ranges |
|  | Drapery controls easily operable and on accessible route of travel |
|  | *Door lock opens with a key easily operable by one hand without tight gripping or twisting |
|  | * Interior security lock easily operable with a closed fist and located within reach ranges |
|  | Windows operable with a closed fist |

## Rooms for Persons with Hearing Impairments

|  | Number of rooms for persons with hearing impairments |
| --- | --- |
|  | Visible alarm system, portable or permanent |
|  | Telephone has volume controls and hearing aid compatible |
|  | Electrical outlet within 48" of the telephone |
|  | Visual notification of incoming calls and door bell |
|  | Closed caption decoders on TV preferred |
|  | Number of portable systems (include visible alarm system, TDDs, visual notification of incoming calls, door bell, and alarm clock) |

*Essential for an accessible meeting

# Hotel/Motel Guest Rooms (Cont’d)

## Bathrooms

|  | *Doorway 32" minimum clear opening width |
| --- | --- |
|  | *Lever or loop-type door handles |
|  | 5' diameter clear floor space in toilet room |
|  | *Grab bars side and back, 33" - 36" above and parallel to floor |
|  | *Grab bars 1-1/2" diameter and 1-1/2" from wall |
|  | *Toilet seat 17" -19" high |
|  | Bottom of mirror, top of shelf, towel, and other dispensers 40" maximum from floor |
|  | 29" clear knee space under basin |
|  | Insulated water pipes under lavatory |
|  | *Lever-type faucets at 17" from front of counter |

## Accessible Tubs and Showers

|  | *Grab bars for tubs _____ showers ______ |
| --- | --- |
|  | *Portable or in tub seat 12" in width, extending width of tub and mounted securely |
|  | *Tub controls operable with a closed fist |
|  | 5' minimum long flexible shower hose with a lower hook or hanger |
|  | Roll in shower 30" x 60" (may include a fold down seat) |
|  | Shower stall 36" x 36" with seat |
|  | Seat mounted 17-19" above the floor, extending depth of stall, located on wall opposite controls, mounted 1-1/2" from shower wall, and no more than 16" in width |
|  | *Shower control operable with a closed fist |

*Essential for an accessible meeting

# Hotel/Motel Guest Rooms (Cont’d)

Please note any problems with access that you observed, or additional comments that will assist you in your selection of the most accessible facility in the area you are considering holding a meeting:

Summary of changes needed prior to the meeting:

### Appendix A

| Number of Accessible Parking Spaces |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Total Parking (Lot or garage) | Total Minimum Number of Accessible Spaces (includes accessible & van) |  |  | Accessible Spaces |  |  | Van Parking Spaces |
| 1 to 25 26 to 50 51 to 75 76 to 100 101 to 150 151 to 200 201 to 300 301 to 400 401 to 500 501 to 999 Over 1,000 | 1 2 3 4 5 6 7 8 9 2 percent of total spaces 20 spaces plus 1 space for every 100 spaces, or fraction thereof, over 1,000 |  |  | 0 1 2 3 4 5 6 7 7 |  |  | 1 1 1 1 1 1 1 1 2 1 of 6 1 of 6 |
| Wheelchair Spaces Required in Assembly Areas |  |  |  |  |  |  |  |
| Capacity of Seating in Assembly Areas |  |  |  |  | Number of Required Wheelchair Spaces |  |  |
| 4 to 25 26 to 50 51 to 300 301 to 500 over 500 |  |  |  |  | 1 2 4 6 6 plus 1 for each 100 over 500 |  |  |
| Number of Accessible Rooms and Roll-in Showers |  |  |  |  |  |  |  |
| Total Number of Rooms |  | Minimum Required Accessible Rooms |  |  |  | Rooms with Roll-in Showers |  |
| 1 to 25 26 to 50 51 to 75 76 to 100 101 to 150 151 to 200 201 to 300 301 to 400 401 to 500 501 to 1,000 Over 1,000 |  | 1 2 3 4 5 6 7 8 9 2% of total 20 plus 1 for every 100 rooms or fraction thereof, over 1,000 |  |  |  | 1 1 2 2 3 4 4 plus 1 for every 100 rooms or fraction thereof, over 400 |  |
| Number of Accessible Rooms for Persons with Hearing Impairments |  |  |  |  |  |  |  |
| Total Number of Rooms |  |  | Minimum Required Number |  |  |  |  |
| 1 to 25 26 to 50 51 to 75 76 to 100 101 to 150 151 to 200 201 to 300 301 to 400 401 to 500 501 to 1,000 Over 1,000 |  |  | 1 2 3 4 5 6 7 8 9 2% of total rooms 20 plus 1 for every 100 rooms, or fraction thereof, over 1,000 |  |  |  |  |

Appendix B

TABLE E104.31

# DWELLING OR SLEEPING UNITS WITH ACCESSIBLE COMMUNICATION FEATURES

| TOTAL NUMBER OF DWELLING OR SLEEPING UNITS PROVIDED | MINIMUM REQUIRED NUMBER OF DWELLING OR SLEEPING UNITS WITH ACCESSIBLE COMMUNICTION FEATURES |
| --- | --- |
| 1 | 1 |
| 2 to 25 | 2 |
| 26 to 50 | 4 |
| 51 to 75 | 7 |
| 76 to 100 | 9 |
| 101 to 150 | 12 |
| 151 to 200 | 14 |
| 201 to 300 | 17 |
| 301 to 400 | 20 |
| 401 to 500 | 22 |
| 501 to 1,000 | 5% of total |
| 1,001 and over | 50 plus 3 for each 100 over 1,000 |